# Cybersecurity Incident Report

| Section 1: Identify the type of attack that may have caused this network interruption |  |
| --- | --- |
| One potential explanation for the website's connection timeout error message is: The logs show that there were two IP addresses that tried to reach the server. No. 47 at 3.144521 seconds/milliseconds IP address 198.51.100.23 (source) started the first three-way handshake to IP address 192.0.2.1(destination) through TCP protocol. The handshake was completed and granted access to the web through HTTP protocol at No. 51 at 3.349457 seconds/milliseconds. The second source IP address 203.0.113.0, No. 52 in the log, started its three-way handshake at 3.390692 seconds/milliseconds to the same destination IP address 192.0.2.1 and sane TCP protocol. It completed its three-way handshake at No. 54 at 3.49316 seconds/milliseconds. Both IP addresses continued to send traffic to the server. IP address 198.51.100.23 sent a SYN packet at 3.544394 seconds/milliseconds (No. 55 in the log) and finished its three-way handshake at 3.730097 seconds/milliseconds, No. 58 through TCP protocol and got connected to HTTP protocol 4.018431 seconds/milliseconds, No. 62. IP address 203.0.113.0 started to send repeated SYN packets that by the time it got to log No. 73 at 6.230548 seconds/milliseconds, the server started to respond RST, ACK. It couldn’t keep up with completing the three-way handshake. It attempted to continue to try to complete the three-way handshake, but responded with the same RST, ACK at No. 80 (at 7.380773 seconds/milliseconds) No. 85 (at 7.680504 seconds/milliseconds), and No. 92 (at 13.967558 seconds/milliseconds). IP address 203.0.113.0 continued to send SYN packets at a rapid pace that the server eventually stopped responding. This event could be a cause of a DOS SYN Flood attack. |  |

Section 2: Explain how the attack is causing the website to malfunction

When website visitors try to establish a connection with the web server, a three-way handshake occurs using the TCP protocol. Explain the three steps of the handshake: 1. The first part of the handshake sends a SYN request for access to the server with the visitor’s IP address. 2. The server then responds to the visitor's IP address with a SYN/ACK, acknowledging the SYN request. 3. The last part of the handshake the visitor’s IP address responds back to the server with an ACK, acknowledging the server’s acknowledgement and approval. Explain what happens when a malicious actor sends a large number of SYN packets all at once: When a malicious actor sends a large number of SYN packets all at once, this overwhelms the server’s ability to respond to the packet request, interrupting the three-way handshake and not giving it time to complete. After a while, the server shuts down with a denial of service (DOS) because it cannot complete its original task. Explain what the logs indicate and how that affects the server: The logs show that there were two IP addresses that tried to reach the server. No. 47 at 3.144521 seconds/milliseconds IP address 198.51.100.23 (source) started the first three-way handshake to IP address 192.0.2.1(destination) through TCP protocol. The handshake was completed and granted access to the web through HTTP protocol at No. 51 at 3.349457 seconds/milliseconds. The second source IP address 203.0.113.0, No. 52 in the log, started its three-way handshake at 3.390692 seconds/milliseconds to the same destination IP address 192.0.2.1 and sane TCP protocol. It completed its three-way handshake at No. 54 at 3.49316 seconds/milliseconds. Both IP addresses continued to send traffic to the server. IP address 198.51.100.23 sent a SYN packet at 3.544394 seconds/milliseconds (No. 55 in the log) and finished its three-way handshake at 3.730097 seconds/milliseconds, No. 58 through TCP protocol and got connected to HTTP protocol 4.018431 seconds/milliseconds, No. 62. IP address 203.0.113.0 started to send repeated SYN packets that by the time it got to log No. 73 at 6.230548 seconds/milliseconds, the server started to respond RST, ACK. It couldn’t keep up with completing the three-way handshake. It attempted to continue to try to complete the three-way handshake, but responded with the same RST, ACK at No. 80 (at 7.380773 seconds/milliseconds) No. 85 (at 7.680504 seconds/milliseconds), and No. 92 (at 13.967558 seconds/milliseconds). IP address 203.0.113.0 continued to send SYN packets at a rapid pace that the server eventually stopped responding.